Федеральное государственное автономное образовательное учреждение высшего образования «Национальный исследовательский университет ИТМО»

«Администрирование систем и сетей»

Лабораторная работа №2

Адресация и маршрутизация IPv4

Выполнил:

Румский Александр Максимович

Группа:

P3407

Преподаватель:

Афанасьев Дмитрий Борисович

г. Санкт-Петербург, 2025 год

Содержание

Топология	3

Конфигурация	4

Смена имен устройств	4

IP адреса физических интерфейсов	4

Настройка IP адресов маршрутизаторов	5

AR1	5

AR2	7

AR3	9

Настройка статических маршрутов между маршрутизаторами	11

AR1	11

AR2	13

AR3	15

Настройка резервных маршрутов	17

Настройка маршрутов между Loopback AR1 и AR1	19

Полезные команды	21

Топология

Конфигурация

Смена имен устройств

<Huawei>system-view

[Huawei]sysname ARN

IP адреса физических интерфейсов

| Маршрутизатор | Интерфейс | Ip/маска |
| --- | --- | --- |
| AR1 | GigabitEthernet0/0/0 | 10.0.12.1/24 |
|  | GigabitEthernet0/0/2 | 10.0.13.1/24 |
| AR2 | GigabitEthernet0/0/0 | 10.0.23.2 24 |
|  | GigabitEthernet0/0/1 | 10.0.12.2/24 |
| AR3 | GigabitEthernet0/0/0 | 10.0.13.3 24 |
|  | GigabitEthernet0/0/1 | 10.0.23.3 24 |

Настройка IP адресов маршрутизаторов

AR1

IP

<AR1>system-view

[AR1]interface GigabitEthernet0/0/2

[AR1-GigabitEthernet0/0/2]ip address 10.0.13.1 24

[AR1-GigabitEthernet0/0/2]quit

[AR1]interface GigabitEthernet0/0/0

[AR1-GigabitEthernet0/0/0]ip address 10.0.12.1 24

[AR1-GigabitEthernet0/0/0]quit

Loopback интерфейс

[AR1]interface LoopBack0

[AR1-LoopBack0]ip address 10.0.1.1 32

Таблицы и пинги

[AR1]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 11       Routes : 11

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.12.0/24  Direct  0    0           D   10.0.12.1       GigabitEthernet

0/0/0

10.0.12.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.0/24  Direct  0    0           D   10.0.13.1       GigabitEthernet

0/0/2

10.0.13.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR1]ping 10.0.12.2

PING 10.0.12.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.2: bytes=56 Sequence=1 ttl=255 time=1000 ms

Reply from 10.0.12.2: bytes=56 Sequence=2 ttl=255 time=130 ms

Reply from 10.0.12.2: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.12.2: bytes=56 Sequence=4 ttl=255 time=60 ms

Reply from 10.0.12.2: bytes=56 Sequence=5 ttl=255 time=30 ms

--- 10.0.12.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/252/1000 ms

[AR1]ping 10.0.13.3

PING 10.0.13.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.13.3: bytes=56 Sequence=1 ttl=255 time=570 ms

Reply from 10.0.13.3: bytes=56 Sequence=2 ttl=255 time=30 ms

Reply from 10.0.13.3: bytes=56 Sequence=3 ttl=255 time=30 ms

Reply from 10.0.13.3: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.13.3: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.13.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/142/570 ms

AR2

IP

<AR2>system-view

[AR2]interface GigabitEthernet0/0/1

[AR2-GigabitEthernet0/0/1]ip addres 10.0.12.2 24

[AR2-GigabitEthernet0/0/1]quit

[AR2]interface GigabitEthernet0/0/0

[AR2-GigabitEthernet0/0/0]ip addres 10.0.23.2 24

[AR2-GigabitEthernet0/0/0]quit

Loopback интерфейс

[AR2]interface LoopBack0

[AR2-LoopBack0]ip address 10.0.1.2 32

Таблицы и пинги

[AR2]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 11       Routes : 11

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.2/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.12.0/24  Direct  0    0           D   10.0.12.2       GigabitEthernet

0/0/1

10.0.12.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.23.0/24  Direct  0    0           D   10.0.23.2       GigabitEthernet

0/0/0

10.0.23.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR2]ping 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=110 ms

Reply from 10.0.12.1: bytes=56 Sequence=2 ttl=255 time=70 ms

Reply from 10.0.12.1: bytes=56 Sequence=3 ttl=255 time=30 ms

Reply from 10.0.12.1: bytes=56 Sequence=4 ttl=255 time=70 ms

Reply from 10.0.12.1: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.12.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/64/110 ms

[AR2]ping 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=130 ms

Reply from 10.0.23.3: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.23.3: bytes=56 Sequence=3 ttl=255 time=20 ms

Reply from 10.0.23.3: bytes=56 Sequence=4 ttl=255 time=50 ms

Reply from 10.0.23.3: bytes=56 Sequence=5 ttl=255 time=20 ms

--- 10.0.23.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/54/130 ms

AR3

IP

<AR3>system-view

[AR3]interface GigabitEthernet0/0/1

[AR3-GigabitEthernet0/0/1]ip address 10.0.23.3 24

[AR3-GigabitEthernet0/0/1]quit

[AR3]interface GigabitEthernet0/0/0

[AR3-GigabitEthernet0/0/0]ip address 10.0.13.3 24

[AR3-GigabitEthernet0/0/0]quit

Loopback интерфейс

[AR3]interface LoopBack0

[AR3-LoopBack0]ip address 10.0.1.3 32

Таблицы и пинги

[AR3-LoopBack0]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 11       Routes : 11

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.3/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.13.0/24  Direct  0    0           D   10.0.13.3       GigabitEthernet

0/0/0

10.0.13.3/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.23.0/24  Direct  0    0           D   10.0.23.3       GigabitEthernet

0/0/1

10.0.23.3/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR3-LoopBack0]ping 10.0.23.2

PING 10.0.23.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.2: bytes=56 Sequence=1 ttl=255 time=90 ms

Reply from 10.0.23.2: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.23.2: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.23.2: bytes=56 Sequence=4 ttl=255 time=60 ms

Reply from 10.0.23.2: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.23.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/58/90 ms

[AR3-LoopBack0]ping 10.0.13.1

PING 10.0.13.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.13.1: bytes=56 Sequence=1 ttl=255 time=30 ms

Reply from 10.0.13.1: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.13.1: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.13.1: bytes=56 Sequence=4 ttl=255 time=50 ms

Reply from 10.0.13.1: bytes=56 Sequence=5 ttl=255 time=60 ms

--- 10.0.13.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/48/60 ms

Настройка статических маршрутов между маршрутизаторами

AR1

[AR1]ip route-static 10.0.1.2 32 10.0.12.2

[AR1]ip route-static 10.0.1.3 32 10.0.13.3

[AR1]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 13       Routes : 13

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.1.2/32  Static  60   0          RD   10.0.12.2       GigabitEthernet

0/0/0

10.0.1.3/32  Static  60   0          RD   10.0.13.3       GigabitEthernet

0/0/2

10.0.12.0/24  Direct  0    0           D   10.0.12.1       GigabitEthernet

0/0/0

10.0.12.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.0/24  Direct  0    0           D   10.0.13.1       GigabitEthernet

0/0/2

10.0.13.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR1]ping -a 10.0.1.1 10.0.1.2

PING 10.0.1.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=255 time=50 ms

Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=255 time=40 ms

Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=255 time=20 ms

--- 10.0.1.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/36/50 ms

[AR1]ping -a 10.0.1.1 10.0.1.3

PING 10.0.1.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.3: bytes=56 Sequence=1 ttl=255 time=80 ms

Reply from 10.0.1.3: bytes=56 Sequence=2 ttl=255 time=70 ms

Reply from 10.0.1.3: bytes=56 Sequence=3 ttl=255 time=30 ms

Reply from 10.0.1.3: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.1.3: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.1.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/54/80 ms

AR2

[AR2]ip route-static 10.0.1.1 32 10.0.12.1

[AR2]ip route-static 10.0.1.3 32 10.0.23.3

[AR2]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 13       Routes : 13

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Static  60   0          RD   10.0.12.1       GigabitEthernet

0/0/1

10.0.1.2/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.1.3/32  Static  60   0          RD   10.0.23.3       GigabitEthernet

0/0/0

10.0.12.0/24  Direct  0    0           D   10.0.12.2       GigabitEthernet

0/0/1

10.0.12.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.23.0/24  Direct  0    0           D   10.0.23.2       GigabitEthernet

0/0/0

10.0.23.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR2]ping -a 10.0.1.2 10.0.1.1

PING 10.0.1.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.1: bytes=56 Sequence=1 ttl=255 time=30 ms

Reply from 10.0.1.1: bytes=56 Sequence=2 ttl=255 time=30 ms

Reply from 10.0.1.1: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.1.1: bytes=56 Sequence=4 ttl=255 time=50 ms

Reply from 10.0.1.1: bytes=56 Sequence=5 ttl=255 time=80 ms

--- 10.0.1.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/48/80 ms

[AR2]ping -a 10.0.1.2 10.0.1.3

PING 10.0.1.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.3: bytes=56 Sequence=1 ttl=255 time=80 ms

Reply from 10.0.1.3: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.1.3: bytes=56 Sequence=3 ttl=255 time=60 ms

Reply from 10.0.1.3: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.1.3: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.1.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/56/80 ms

AR3

[AR3]ip route-static 10.0.1.1 32 10.0.13.1

[AR3]ip route-static 10.0.1.2 32 10.0.23.2

[AR3]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 13       Routes : 13

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Static  60   0          RD   10.0.13.1       GigabitEthernet

0/0/0

10.0.1.2/32  Static  60   0          RD   10.0.23.2       GigabitEthernet

0/0/1

10.0.1.3/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.13.0/24  Direct  0    0           D   10.0.13.3       GigabitEthernet

0/0/0

10.0.13.3/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.23.0/24  Direct  0    0           D   10.0.23.3       GigabitEthernet

0/0/1

10.0.23.3/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR3]ping -a 10.0.1.3 10.0.1.1

PING 10.0.1.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.1: bytes=56 Sequence=1 ttl=255 time=40 ms

Reply from 10.0.1.1: bytes=56 Sequence=2 ttl=255 time=70 ms

Reply from 10.0.1.1: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.1.1: bytes=56 Sequence=4 ttl=255 time=50 ms

Reply from 10.0.1.1: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.1.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/48/70 ms

[AR3]ping -a 10.0.1.3 10.0.1.2

PING 10.0.1.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=255 time=20 ms

Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=255 time=60 ms

Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.1.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/38/60 ms

Настройка резервных маршрутов

[AR1]ip route-static 10.0.1.2 32 10.0.13.3 preference 100

[AR2]ip route-static 10.0.1.1 32 10.0.23.3 preference 100

[AR1]interface GigabitEthernet0/0/0

[AR1-GigabitEthernet0/0/0]shutdown

[AR1]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 10       Routes : 10

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.1.2/32  Static  100  0          RD   10.0.13.3       GigabitEthernet

0/0/2

10.0.1.3/32  Static  60   0          RD   10.0.13.3       GigabitEthernet

0/0/2

10.0.13.0/24  Direct  0    0           D   10.0.13.1       GigabitEthernet

0/0/2

10.0.13.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR2]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 10       Routes : 10

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Static  100  0          RD   10.0.23.3       GigabitEthernet

0/0/0

10.0.1.2/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.1.3/32  Static  60   0          RD   10.0.23.3       GigabitEthernet

0/0/0

10.0.23.0/24  Direct  0    0           D   10.0.23.2       GigabitEthernet

0/0/0

10.0.23.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR1]tracert -a 10.0.1.1 10.0.1.2

traceroute to  10.0.1.2(10.0.1.2), max hops: 30 ,packet length: 40,press CTRL_C

to break

1 10.0.13.3 30 ms  50 ms  20 ms

2 10.0.23.2 80 ms  50 ms  40 ms

Настройка маршрутов между Loopback AR1 и AR1

[AR1]undo ip route-static 10.0.1.2 255.255.255.255 10.0.12.2

[AR1]undo ip route-static 10.0.1.2 255.255.255.255 10.0.13.3

[AR1]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 12       Routes : 12

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.1.1/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.1.3/32  Static  60   0          RD   10.0.13.3       GigabitEthernet

0/0/2

10.0.12.0/24  Direct  0    0           D   10.0.12.1       GigabitEthernet

0/0/0

10.0.12.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.0/24  Direct  0    0           D   10.0.13.1       GigabitEthernet

0/0/2

10.0.13.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR1]ip route-static 0.0.0.0 0 10.0.12.2

[AR1]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 13       Routes : 13

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

0.0.0.0/0   Static  60   0          RD   10.0.12.2       GigabitEthernet

0/0/0

10.0.1.1/32  Direct  0    0           D   127.0.0.1       LoopBack0

10.0.1.3/32  Static  60   0          RD   10.0.13.3       GigabitEthernet

0/0/2

10.0.12.0/24  Direct  0    0           D   10.0.12.1       GigabitEthernet

0/0/0

10.0.12.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.0/24  Direct  0    0           D   10.0.13.1       GigabitEthernet

0/0/2

10.0.13.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[AR1]ping -a 10.0.1.1 10.0.1.2

PING 10.0.1.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=255 time=380 ms

Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.1.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/110/380 ms

Полезные команды

display ip interface brief – ip адреса интерфейсов

display ip routing-table – таблица маршрутизации